Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 CSI@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Bullet thermal QVGA 24° 30Hz

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

Bullet thermal QVGA 24° 30Hz

- GENERAL

  - SUMMARY

## Section includes a bullet thermal camera that is using the latest Uncooled microbolometer / VOx (vanadium oxide) sensor to detect the thermal emissions and visualizes subtle temperature difference within a scene. The bullet thermal camera is integrated with a 9 mm lens, with a 24° horizontal field of view, IP66 / IP67 and IK10-rated housing, with H.264 and H.265 encoding, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

## Related requirements

      - 27 15 00 Communications Horizontal Cabling

      - 27 20 00 Data Communications

      - 28 05 00 Common Work Results for Electronic Safety and Security

      - 28 06 20 Schedules for Video Surveillance

      - 28 16 15 Access Control Interfaces to Video Surveillance

      - 28 23 00 Video Management System

      - 28 31 31.15 Intrusion Detection Interfaces to Video Surveillance

      - 28 51 15.13 Information Interfaces to Video Surveillance Systems

## Sustainability

      - The fixed network camera shall be manufactured in compliance with ISO 9001 and ISO 14001.

      - The fixed network camera shall be compliant with the EU Directives 2011/65/EU (RoHS) and 2012/19/EU (WEEE).

      - Packaging shall be compliant with EU Directive 94/62/EC (amended by 2004/12/EC).

      - The fixed network camera shall be compliant with the EU regulation 1907/2006 (REACH).

  - REFERENCES

    - Abbreviations

      - AI – Artificial Intelligence

      - ARP – Address Resolution Protocol

      - DHCP - Dynamic Host Configuration Protocol

      - DNS - Domain Name Server

      - DDNS – Dynamic DNS

      - DORI - Detect, Observe, Recognize, Identify

      - fps - frames per second

      - HEVC – High Efficiency Video Coding

      - HTTPS – Secure Hypertext Transfer Protocol

      - ICMP - Internet Control Message Protocol

      - IP - Internet Protocol

      - IR – InfraRed

      - JPEG - Joint Photographic Experts Group

      - mDNS – Multicast DNS

      - MP - Megapixel

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PKI - Public Key Infrastructure

      - PoE - Power over Ethernet

      - RTCP – Real-Time Control Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - TCP - Transmission Control Protocol

      - TPM - Trusted Platform Module

      - TLS – Transport Layer Security

      - UDP - User Datagram Protocol

    - Reference Standards

      - Network

        - IEEE 802.3af / 802.3at Type 1, Class 3 Ethernet Standards

      - Cybersecurity

        - IEC 62443-4-2 and IEC 62443-4-1

        - UL 2900-2-3

        - Common Criteria EAL6+ (certified secure element)

      - Video

        - H.264 (ISO/IEC 14496-10)

        - H.265/HEVC

        - MJPEG

      - ONVIF

        - IEC 62676-2-3

        - EN 60839-11-31

        - EN IEC 62676-2-31

        - EN IEC 62676-2-32

      - Emissions

        - EN 55032 (Class A)

        - CISPR32 (Class A)

        - ICES-003 (Class A)

        - FCC Part 15 Subpart B Class A

        - IEC 61000-6-4

        - EN IEC 61000-6-4

        - RSS-Gen

        - VCCI-CISPR 32

      - Immunity

        - EN 55035

        - CISPR35

        - IEC 61000-6-2

        - EN IEC 61000-6-2

        - EN 50130-4

        - EN 50121-4

      - Safety

        - AS/NZS 62368.1

        - CSA C22.2 No. 62368-1

        - EN IEC 62368-1

        - IEC 62368-1

        - J62368-1

        - SASO-IEC 62368-1

        - UL 62368-1

      - Environmental

        - RoHS EU, 2011/65/EU

        - EN IEC 63000

        - WEEE EU, 2012/19/EU

        - Packaging EU, 94/62/EC

        - DIN EN 62321:2012-10

      - Reliability

        - EN 60068-2-1

        - EN 60068-2-2

        - EN 60068-2-6

        - EN 60068-2-18

        - EN 60068-2-27

        - EN 60068-2-30

        - EN 60068-2-42

        - EN 60068-2-52

        - EN 60068-2-75

        - EN 60068-2-78

      - Corrosion resistance

        - ISO 14993

      - Impact protection

        - EN 62262 (IK10)

      - Ingress protection

        - EN 60529 (IP66/67)

        - UL50E Type 4X

      - Shock / vibration protection.

        - NEMA TS2 (section 2.2.8 - section 2.2.9)

      - Regulatory compliance

        - NDAA compliant

        - TAA compliant

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

      - Shop Drawings

      - Cyber hardening guidance

    - Closeout Submittals

      - Final listing of devices and settings

      - System test results

      - Statement of compliance with Manufacturer Cyber Hardening Guidelines

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 585 223 9180

Email: technical.support@us.bosch.com

    - Product Model:	NBT-8700-F09QF

  - GENERAL DESCRIPTION

    - The camera shall be a QVGA bullet thermal camera with 9 mm lens, IP66/ IP67 and IK10-rated housing, H.264 and H.265 encoding, a frequency of 30 Hz, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

    - The bullet thermal camera shall possess the following primary characteristics:

      - H.264, H.265, and MJPEG compression

      - Deep learning based detection and tracking of person and vehicle

      - AI-based autocalibration

      - Bi-directional audio

      - Multiple data security elements including Trusted Platform Module (TPM), PKI, 4096-bit RSA encryption, and update capability for future software security functions

      - Available posting to cloud-based services

      - Remote camera management

      - Mobile device camera setup and configuration

      - IP66, IP67, UL Type 4X, and IK10 environmental ratings

      - Power:	24 VAC, 12 - 26 VDC, or PoE IEEE 802.3af / 802.3at Type 1, Class 3

      - The fixed network camera shall provide a 12 VDC 50 mA power output.

  - ADDITIONAL VIDEO/CAMERA SPECIFICATIONS

    - The thermal sensor shall possess the following primary characteristics:

      - Sensor type:                                Uncooled microbolometer / VOx sensor

      - Spectral range:                            8 μm - 14 μm

      - Resolution:                                  QVGA (320 x 240 pixels) minimum

      - Pixel pitch:			12 μm or larger

      - Frequency:			30 Hz minimum

      - Sensitivity (NETD)		<30 mK at 25 °C

    - The thermal lens shall possess the following primary characteristics:

      - Focal length: 			9 mm

      - Aperture:			0.8/F

      - Horizontal field of view:		24°

      - Vertical field of view:		18°

      - Focus:				Fixed focus, athermalized

      - Minimum focus distance: 	0.5 m

    - Video streaming:

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall provide one M-JPEG and four H.264/H.265 independently configurable streams.

      - The fixed network camera shall deliver QVGA (320x 240pixels) video, at rates up to 30 images per second.

    - Additional video functions:

      - The fixed network camera shall offer video watermarking capabilities.

      - The fixed network camera shall provide display stamping.

      - The fixed network camera shall be capable of displaying its location information.

      - The fixed network camera shall be capable of adjusting below picture setting:

        - Auto contrast mode

        - Contrast Level

        - Brightness Level

        - Sharpness Level

        - Histogram equalization

        - Custom contrast region

        - Noise filter

        - Image rotation

      - The fixed network camera shall include 12 thermal modes, including

        - White hot

        - Black hot

        - Red light

        - Arcus

        - Inferno

        - SoftLight

        - Sunset

        - Mermoriam

        - Flamma Arcticus

        - Ocean

        - Rainbow

        - Iron Black

      - The fixed network camera shall allow for predefined contrast region including:

        - Full screen

        - Upper

        - Lower

        - Left

        - Right

        - Center

        - Custom

    - Privacy masks

      - The fixed network camera shall offer the ability to define up to 8 privacy masks.

      - The fixed network camera shall offer multiple user defined privacy masks patterns, including:

        - Black

        - White

        - Gray

        - Auto

        - Custom

  - System Features

    - The local SD storage shall possess the following primary characteristics:

      - Micro SD card slots: 2

      - Micro SDHC or Micro SDXC, up to 2TB

    - The inputs and outputs shall possess the following primary characteristics:

      - The fixed network camera shall provide two (2) alarm inputs.

      - The fixed network camera shall provide one (1) alarm output.

    - The audio shall possess the following primary characteristics:

      - The fixed network camera shall provide one (1) audio line out.

      - The fixed network camera shall provide switchable audio input. This includes one (1) line-in level or one (1) microphone-in level audio input.

      - The fixed network camera shall offer the G.711 8 kHz, L16 16 kHz, AAC-LC 80 kbps 16 kHz, and AAC-LC 48 kbps 16 kHz compression rates.

      - The fixed network camera shall provide full duplex and half duplex streaming.

    - The fixed network camera shall be capable of disabling audio input permanently with a license code.

  - Video analytics

    - The fixed network camera shall offer comprehensive video content analysis capabilities, including detection and classification of persons and vehicles.

    - The fixed network camera shall offer embedded IVA Pro Perimeter for reliable deep learning-based detection.

    - The fixed network camera shall offer embedded deep learning-based detection and video analytics that eliminate dedicated PCs and associated software maintenance.

    - The fixed network camera shall be capable of running 11 different video analytics rules simultaneously.

    - The fixed network camera shall offer video analytics rule configuration, including:

      - Any moving object

      - Object in field

      - Entering field

      - Leaving field

      - Crossing line

      - Counter

      - Occupancy

      - Loitering

      - Following route

      - Condition change

      - Similarity search

    - The fixed network camera shall support automatic switching of video analytics types according to a predefined schedule.

    - The fixed network camera shall be capable of generating metadata, including object speed, geolocation, height, width, and classification to be recorded with the video.

    - The fixed network camera shall be capable of detecting and classifying up to 64 objects.

    - The fixed network camera shall support machine learning to enable users to classify customized objects.

    - The fixed network camera shall offer a tampering detection.

    - The fixed network camera shall be capable of detecting an upright person at a distance of up to 124 m (407 ft), based on a target height of 1.8 m (5.9 ft), using video analytics.

  - NETWORK

    - The fixed network camera shall support iSCSI devices to allow the network-enabled camera to stream video directly to an iSCSI RAID array.

    - The fixed network camera shall conform to the ONVIF profiles S, G, T and M standard.

    - The fixed network camera shall support the following protocols:

      - IPv4; IPv6; UDP; TCP; HTTP; HTTPS; RTP/RTCP; IGMP V2/V3; ICMP; ICMPv6; RTSP; FTP; ARP; DHCP; APIPA (Auto-IP, link local address); NTP (SNTP); SNMP (V1, MIBII); SNMP (V3, MIBII); 802.1x, EAP/TLS; DNS; DNSv6; DDNS (DynDNS.org, selfHOST.de, no-ip.com); SMTP; iSCSI; DiffServ (QoS); LLDP; SOAP; CHAP; Digest authentication

    - The data security shall possess the following primary characteristics.

      - Crypto coprocessor: 		RSA 4096 bit; AES/CBC 256 bit

      - PKI				X.509 certificates

      - End-to-end encryption: 		TLS 1.2; AES 256; AES 128; TLS 1.3

      - Encryption 			TLS 1.2; AES 256; AES 128; TLS 1.3

      - Video Authentication:		MD5; SHA-1; SHA-256; Checksum

      - Local storage encryption:	XTS-AES

      - Firmware protection:		Signed firmware; Secure boot

  - ELECTRICAL

    - The fixed network camera shall support the following redundant power options:

      - Input options:

        - PoE (IEEE 802.3af / 802.3at Type 1, Class 3)

        - 12-26 VDC

        - 24 VAC

    - The fixed network camera shall default to use PoE power.

    - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC/12 VDC power supply if power from the PoE power supply is lost.

    - The fixed network camera shall be equipped with an integrated window heater.

  - MECHANICAL AND ENVIRONMENTAL

    - The construction material shall possess the following primary characteristics:

      - Housing: Aluminum with dehumidifying membrane and waterproof connection area

    - The finish options shall be:

      - White

    - The impact resistance shall be:

      - IK10

    - The dimensions (Ø x D) shall be:

      - 148 mm x 352 mm (5.82 in x 13.85 in)

    - The temperature shall be:

      - Operating:			-40°C to 55°C (-40°F to 131°F)

      - Storage:			-40°C to 70ºC (-40°F to 158ºF)

    - The relative humidity (non-condensing) shall be:

      - Operating			5 to 95%

    - The ingress protection shall be:

      - IP66

      - IP67

    - The mounting shall be:

      - Surface-mounted

  - ACCESSORIES

    - The manufacturer shall offer accessories to enable the mounting of the dome camera in the following circumstances:

      - Pole mount adapter

      - Corner mount adapter

    - Other accessories shall be available:

      - Conduit back box

      - Surveillance cabinet – 24 VAC, 120 VAC, 230 VAC

      - Ethernet to SFP interface kit

      - Wireless installation dongle – EU, NA

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the contractor prior to camera installation.

    - Before permanent installation of the system, the contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

  - STORAGE

    - Products shall be stored in an environment where temperature and humidity are in the range specified by the manufacturer.

END OF SECTION

202507310902