Product Tests Report

Product name:		AUTODOME inteox 7000i Camera

Model number and description:

Pendant outdoor model: NDP-7602-Z30, PTZ 2MP 30x clear IP66 pendant

Pendant outdoor (IK) model: NDP-7602-Z30K, PTZ 2MP 30x clear IK10 pendant
Indoor in-ceiling model: NDP-7602-Z30C, PTZ 2MP 30x clear IP54 in-ceiling

Indoor in-ceiling model: NDP-7602-Z30CT, PTZ 2MP 30x tinted IP54 in-ceiling

Pendant outdoor model: NDP 7602-Z30-OC, PTZ 2MP 30x clear pendant OC
Pendant outdoor model:  VG5-ITS1080P-30X7 PTZ 2MP 30x TS-2 pendant ITS

The above-mentioned Bosch Security Systems product has been tested in accordance and was found to comply with the tests listed below which were carried out during the development phase of the product. Please note that for some tests, test conditions exceed the range that Bosch recommends for effective continuous operation of the camera.

Data subject to change without notice.

ENVIRONMENTAL TEST

| BS EN 50130-5:1999 Alarm systems Part 5: Environmental test methods | Specific Test Description (Temperatures stated are outside of the camera housing) | Comments | Passed |
| --- | --- | --- | --- |
| Dry heat (operational) EN 60068-2-2:2007 | Pendant: Temp. +70 °C, duration 48 hours In-ceiling: Temp. +55 °C, duration 48 hours |  | Yes |
| Dry heat (endurance) EN 60068-2-2:2007 | Temp. +70 °C, duration 21 days. | Outdoor models only. | Yes |
| Cold (operational) EN 60068-2-1:2007 | Pendant: Temp. -40 °C, duration 48 hours In-ceiling: Temp. -10 °C, duration 48 hours |  | Yes |
| Cold start test | Power off, soak overnight (12 hours) @ -40 °C, power up unit, unit must power up & function normally within < 30 minutes. | Outdoor models only. | Yes |
| Damp heat, steady state (endurance) EN 60068-2-78:2001 | Temperature +40ºC, Relative Humidity 93%, Duration 21 days. |  | Yes |
| Damp heat, cyclic (operational) EN 60068-2-30:2005 | Pendant: Temperature +25ºC to +55ºC, Relative humidity 93%, 2 cycles. |  | Yes |
| Damp heat, cyclic (endurance) EN 60068-2-30:2005 | Temperature +25ºC to +55ºC, Relative humidity 93%, 6 cycles. | Outdoor models only. | Yes |
| Ingress Protection Rating- IP6X IEC 60529, Edition 2.2, CLAUSE 13.4 | No dust ingress; complete protection against contact. | Outdoor models only. | Yes |
| Water ingress (operational)- IPX6 IEC 60529, Edition 2.2, CLAUSE 14.2.6 | At least 3 minutes Water volume: 100 liters per minute | Outdoor models only. | Yes |
| Ingress Protection Rating- IP5X IEC 60529, Edition 2.2, CLAUSE 13.4 | Dust protected (Ingress of dust is not prevented completely, but it must not enter in sufficient quantity to interfere with the satisfactory operation of the equipment; complete protection against contact) | Indoor models only | Yes |
| Water ingress (operational)- IPX4 IEC 60529, , Edition 2.2, CLAUSE 14.2.4 | 5 minutes Water volume: 10 liters per minute Pressure: 100 kPa | Indoor models only | Yes |
| NEMA 4X | Hose down Test Additional Corrosion Test Crushing Resistance Test Mold Stress-Relief Distortion Test Gasket aging Test Resistance To Impact Test | Pendant outdoor (IK) model only | Yes |
| Vibration sinusoidal (operational) IEC 60068-2-6: Edition 7.0: 2007 | Freq. Range 10-150Hz, 0.5g, 3 axes, sweep rate 1 octave/m 1 sweep/ axis |  | Yes |
| Vibration sinusoidal (endurance) IEC 60068-2-6 Edition 7.0: 2007 | Freq. Range 10-150Hz, 1.0g, 3 axes, sweep rate 1 octave/min 20 sweeps/ axis |  | Yes |
| Shock (operational) IEC 60068-2-27 Edition 4.0:2008 | Half sine wave 6ms, 10.0g, 6 number of shocks, 3 pulses per direction |  | Yes |
| Impact (operational)- IK10 IEC 62262 First Edition Dated 2002-02 | Impact energy 20.0 Joule, 3 impacts per point | Pendant outdoor (IK) model only | Yes |
| Wind speed (150mph wind tunnel test) | Wind Tunnel Test 150mph for 1.0 hours. |  | Yes |
| Sound level (audible noise) | < 45 dB |  | Yes |
| Transportation Tests (ISTA Procedure 2A) | Atmospheric condition Test Compression Test Drop Test- 970 mm Vibration- Frequency 4Hz, 240 cycles per min, duration 60min. |  | Yes |

ADDITIONAL CERTIFICATIONS

| Functional tests description | Description | Comments | Passed |
| --- | --- | --- | --- |
| Conformant with NTCIP protocols 1205, 2301, 1101:1996, 2202:2001, 2101:2001, 2102:2003 |  |  | Yes |

ADDITIONAL ENVIRONMENTAL – FUNCTIONAL BOSCH TESTS

| Environmental test methods | Specific Test Description | Comments | Passed |
| --- | --- | --- | --- |
| MTBF calculation of used components Based on: for electronics, MIL-HDBK-217FN1, GB, GC, 25C. | MTBF > 114,014 hours |  | Yes |
| NEMA TS-2 Traffic Controller Assemblies w/NTCIP Requirements | TS 2 Section 2.2.7 Transients, Temperature, Voltage, and Humidity | Outdoor models only. | Yes |
| NEMA TS-2 Traffic Controller Assemblies w/NTCIP Requirements | TS 2 Section 2.2.8 Vibration – Resonant Search Conduct the resonant frequency search from 5 to 30Hz and back within a period of 12-1/2 minutes in each of the three mutually perpendicular planes | Outdoor models only. | Yes |
| NEMA TS-2 Traffic Controller Assemblies w/NTCIP Requirements | TS 2 Section 2.2.8 Vibration – Endurance Test Vibrate the test unit in each planes at its resonant Frequency for a period of 1 hour at an amplitude resulting in 0.5g acceleration | Outdoor models only. | Yes |
| NEMA TS-2 Traffic Controller Assemblies w/NTCIP Requirements | TS 2 Section 2.2.9 Shock (Impact) Test 10 drops the test unit in each planes from the calibrated height with shock force from 9g to 11g | Outdoor models only. | Yes |

Approvals Safety, EMC and Environmental

| Specific Approval | Description | Comments | Passed |
| --- | --- | --- | --- |
| EMC Europe |  |  | Passed |
| EN 55032:2015/AC:2016 | Electromagnetic compatibility of multimedia equipment - Emission Requirements, Class A. |  | Yes |
| EN 55024:2010 + A1:2015 | Information technology equipment - Immunity characteristics - Limits and methods of measurement |  | Yes |
| EMC USA |  |  | Passed |
| FCC 47 CFR Part 15 Subpart B | Conducted + Radiated Emission based on VERIFICATION procedure, Class A |  | Yes |
| Safety Europe |  |  |  |
| EN 62368-1 | Audio/video, information and communication technology equipment |  | Yes |
| EN 60950-1 | Information technology equipment — Safety — Part 1: General requirements |  | Yes |
| EN 60950-22 | Information technology equipment — Safety — Part 22: Equipment to be installed outdoors | Outdoor models only. | Yes |
| Safety USA + Canada |  |  |  |
| UL 62368-1 | Audio/video, information and communication technology equipment |  | Yes |
| CSA-C22.2 No. 62368-1-14 | Audio/video, information and communication technology equipment |  | Yes |
| Environmental |  |  |  |
| Restriction of Hazardous Substances | 2011/65/EU (RoHS) complaint |  | Yes |
| Restriction of Hazardous Substances | REACH (no substances prohibited in REACH annex XVII and no information about possible SVHCs contained). |  | Yes |

The product is produced by a manufacturing organization which is certified on ISO9001 and ISO14001 standards.