August 2025

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 security.sales@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2004 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

SECTION 28 23 29

VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

BOSCH FLEXIDOME corner 7100i IR

- – GENERAL

  - SUMMARY

    - Related Sections

      - Section [28 23 13 – Video Surveillance Control and Management Systems].

      - Section [28 23 16 – Video Surveillance Monitoring and Supervisory Interfaces].

      - Section [28 23 19 – Digital Video Recorders and Analog Recording Devices].

      - Section [28 23 23 – Video Surveillance Systems Infrastructure].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - EMC – Emissions

      - EN 55032 (Class A) - Electromagnetic compatibility of multimedia equipment - Emission requirements

      - CFR 47 FCC part 15 (Class A) Code of Federal Title 47 – Telecommunication Chapter I – Federal communications commission, Subchapter A – General, Part 15 - Radio frequency devices

      - EN IEC 61000-3-2 Electromagnetic compatibility (EMC) – Part 3-2: Limits – Limits for harmonic current emissions (equipment input current ≤ 16 A per phase)

      - EN IEC 61000-3-3 Electromagnetic compatibility (EMC) Part 3: Limits – Section 3: Limitation of voltage fluctuations and Flicker in low-voltage supply systems for equipment with rated current < 16A

      - EN-IEC 61006-6-4 Electromagnetic compatibility (EMC) - Part 6-4: Generic standards - Emission standard for industrial environments

      - ICES-003 Issue 7 Information Technology Equipment (including Digital Apparatus)

      - ANSI C63.4-2014 American National Standard for Methods of Measurement of Radio-Noise Emissions from Low-Voltage Electrical and Electronic Equipment in the Range of 9 kHz to 40 GHz

      - AS/NZS CISPR 32:2015 + A1:2020 Electromagnetic compatibility of multimedia equipment - Emission requirements

      - VCCI-CISPR 32:2016 EMC Emission Testing of Multimedia Equipment

    - EMC – Immunity

      - EN 50121-4	Railway applications - Electromagnetic compatibility - Part 4: Emission and immunity of the signaling and telecommunications apparatus

      - EN 50130-4	Alarm systems - Part 4: Electromagnetic compatibility - Product family standard: Immunity requirements for components of fire, intruder, hold up, CCTV, access control and social alarm systems

      - EN 61000-4-2 Electromagnetic compatibility (EMC) - Part 4-2: Testing and measurement techniques - Electrostatic discharge immunity test

      - EN 61000-4-3 Electromagnetic compatibility (EMC) Part 4-3: Testing and measurement techniques – Radiates, radio-frequency, electromagnetic field immunity test

      - EN 61000-4-4 Electromagnetic compatibility (EMC) Part 4-4: Testing and measurement techniques - Electrical fast transient/burst immunity test

      - EN 61000-4-5 Electromagnetic compatibility (EMC) Part 4-5: Testing and measurement techniques – Surge immunity test

      - EN 61000-4-6 Electromagnetic compatibility (EMC) Part 4-6: Testing and measurement techniques - Immunity to conducted disturbances, induced by radio-frequency fields

      - EN 61000-4-8 Electromagnetic compatibility (EMC) Part 4-8: Testing and measurement techniques - Power frequency magnetic field immunity test

      - EN 61006-4-11 Electromagnetic compatibility (EMC) - Part 4-11: Testing and measurement techniques - Voltage dips, short interruptions and voltage variations immunity tests

      - EN 55035 Electromagnetic compatibility (EMC) of multimedia equipment — Immunity Requirements

    - Environmental

      - EN 50130-5 (Class II) Alarm systems Part 5: Environmental test methods

      - EN 60068-2-6 Environmental testing Part 2-6: Tests - Test Fc: Vibration (sinusoidal)

      - EN 60068-2-27 Environmental testing Part 2: Tests - Test Ea and guidance: Shock

      - EN IEC 63000 (RoHS) Technical documentation for the assessment of electrical and electronic products with respect to the restriction of hazardous substances

      - RoHS EU, 2011/65/EU and 2015/863/EU - Directives of the European Parliament and of the Council as regards the list of restricted substances and the restriction of the use of certain hazardous substances in electrical and electronic equipment

      - WEEE EU, 2012/19/EU - Directive of the European Parliament and of the Council of 4 July 2012 on waste electrical and electronic equipment (WEEE)

      - California Proposition 65    Safe Drinking Water and Toxic Enforcement Act

    - Safety

      - EN 62368-1	Audio/video, information, and communication technology equipment - Part 1: Safety requirements

      - EN 60950-22 Information technology equipment - Safety - Part 22: Equipment to be installed outdoors

      - UL 60950-22 Information technology equipment - Safety - Part 22: Equipment to be installed outdoors

      - CSA/UL 62368-1:2019	Audio/video, information, and communication technology equipment - Part 1: Safety requirements

    - Image performance

      - IEC 62676-5	Video surveillance systems for use in security applications - Part 5: Data specifications and image quality performance for camera devices

    - ONVIF conformance

      - EN 50132-5-2	Alarm systems - CCTV surveillance systems for use in security applications - Part 5-2: IP Video Transmission Protocols

      - EN 62676-2	Video surveillance systems for use in security applications

    - Marks

      - CE, cULus, RCM, WEEE, VCCI, CMIM, UKCA, FCC

    - Impact protection

      - EN 62262 (IK10+ 50J) Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts (IK code)

    - Ingress protection

      - EN 60529 (IP66) Degrees of protection provided by enclosures (IP Code)

      - UL50E (NEMA 250 Type 4X) Enclosures for Electrical Equipment, Environmental Considerations 1st edition

    - IR lighting

      - EN IEC 62471 Photobiological safety of lamps and lamp systems

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Surveillance Remote Devices

    - Performance Requirements

      - The fixed network camera shall offer a Corner-mount and anti-ligature (no-grip) design for maximum safety with quick and easy installation and set-up.

      - The fixed network camera shall use a high performance 1/1.8-inch CMOS sensor type with up to 6MP resolution.

      - The fixed network camera shall support the following dual, redundant power options:

        - Input options:

          - PoE (IEEE 802.3af / 802.3at Type 1, Class 3)

          - 24 VAC

          - 12 VDC

        - The fixed network camera shall default to use PoE power. PoE and auxiliary power can be connected simultaneously for redundant operation.

        - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC / 12 VDC power supply if power from the PoE power supply is lost.

      - The fixed network camera shall be optimized for all lighting conditions with integrated intelligent IR, covering up to 15 m (50 ft).

      - The fixed network camera shall offer invisible IR (940nm)

      - The fixed network camera shall offer a High Dynamic Range of 120 dB for clear images in extreme high-contrast environments.

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall offer embedded Intelligent Video Analytics (IVA) that eliminates dedicated PCs and associated software maintenance.

      - The fixed network camera shall offer Intelligent Audio Analytics, providing alarm functionality for audio events based on sound characteristics.

      - The fixed network camera shall conform to the ONVIF Profile S, G, M and T to provide interoperability with other conformant systems.

      - The fixed network camera shall offer configurable multi streaming with individually configurable 6MP streams.

      - The fixed network camera shall offer an integrated microphone array with 3 digital MEMS audio sensors.

      - The fixed network camera shall offer bi-directional audio input/output.

      - The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10+ 50J (IK11) impact resistance.

        - Conform to the NEMA type 4X standard.

        - Support a temperature range of -10 ºC to +50 ºC (-14 ºF to +122 ºF) for continuous operation with IR on.

        - Support a temperature range of -30 ºC to +70 ºC (-22 ºF to +158 ºF) for storage.

  - SUBMITTALS

    - Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Shop Drawings; include

      - System device locations on architectural floor plans.

      - Full Schematic of system, including wiring information for all devices.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - System device locations on architectural floor plans.

      - Wiring and connection diagram.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [20] years experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - Listed by cULus.

      - Complies to FCC, CE and UL product specific requirements Test methods are in accordance to international standards.  Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers, and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

    - Bosch provides off-the-shelf availability for our top selling products and same-day or 24-hour shipping.

  - WARRANTY

    - Provide manufacturer’s warranty covering 5 years for replacement and repair of defective equipment.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

[Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450, USA

Phone: + 1 800 289 0096

Fax: + 1 585 223 9180

security.sales@us.bosch.com

www.boschsecurity.us]

[Bosch Security Systems B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: + 31 40 2577 284

Fax: +31 40 2577 330

emea.securitysystems@bosch.com

www.boschsecurity.com]

[Asia-Pacific

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

apr.securitysystems@bosch.com

www.boschsecurity.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - BOSCH FLEXIDOME corner 7100i IR camera

    - General Characteristics:

      - The fixed network camera shall provide a 1/1.8-inch CMOS image sensor with the following minimum capabilities:

        - 2816x2112 effective picture elements.

        - Sensitivity of 30 IRE, down to 0.072 lux (color), 0.014 lux, 0 with IR (mono)

        - High Dynamic Range (HDR) multi-exposure

      - The fixed network camera shall offer a focal length of 2.5 mm.

      - The camera shall offer 131.1° Horizontal and 96.5° Vertical field of views for complete scene coverage with no blind spots and high speeds

      - The camera shall offer full frame rate recording (30fps)

      - .

      - The camera shall use the DORI (Detect, Observe, Recognize, Identify) standard system to distinguish persons or objects within a covered area.

      - The fixed network camera shall provide direct network connection using H.265, H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The fixed network camera shall offer a High Dynamic Range of 120 dB for clear images in extreme high-contrast environments. The fixed network camera shall offer bi-directional audio input/output.

      - The fixed network camera shall offer the ability to define up to 8 privacy masks that will automatically recolor to match the scene color and prohibit areas of the field of view from being seen.

      - The fixed network camera shall offer advanced Intelligent Video Analysis (IVA) with automated calibration

      - The fixed network camera shall offer an integrated microphone array with 3 digital MEMS audio sensors.

      - The fixed network camera shall offer a visible activity LED to indicate the camera is recording

      - The fixed network camera shall support the following dual, redundant power options:

        - Input options:

          - PoE (IEEE 802.3af / 802.3at Type 1, Class 3)

          - 24 VAC

          - 12 VDC

        - The fixed network camera shall default to use PoE power. PoE and auxiliary power can be connected simultaneously for redundant operation.

        - The fixed network camera shall switch with no interruption on camera operation to the 24 VAC / 12 VDC power supply if power from the PoE power supply is lost.

      - The fixed network camera shall be able to be direct corner mounted to the ceiling of any room.

      - The fixed network camera shall be capable of operating in an outdoor environment.

      - The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10+ 50J (IK11) impact resistance.

        - Conform to the NEMA type 4X standard.

        - Support a temperature range of -10 ºC to +50 ºC (-14 ºF to +122 ºF) for continuous operation with IR on.

        - Support a temperature range of -30 ºC to +70 ºC (-22 ºF to +158 ºF) for storage.

      - The fixed network camera shall support the following languages:

        - English

        - Czech

        - Dutch

        - French

        - German

        - Italian

        - Polish

        - Portuguese (Portugal)

        - Russian

        - Spanish

        - Japanese

        - Chinese (Simplified)

      - The fixed network camera’s country of origin shall be:

        - Thailand for NCE-7703-FK

        - Taiwan for NCE-7703-FK-GOV

    - Imaging

      - The fixed network camera shall offer a 1/1.8-inch (6MP) CMOS image sensor.

      - The fixed network camera shall offer a minimum effective number of pixels of:

        - 2816x2112 effective picture elements.

      - The fixed network camera shall offer a High Dynamic Range (HDR) of 120 dB.

      - The fixed network camera shall offer a Sodium Vapor White Balance mode that automatically compensate for light from a sodium vapor lamp to restore objects to their true color.

      - The fixed network camera shall offer an anti-fog image feature that assists the camera in registering a usable image through the heavy fog.

    - Image Processing

      - The fixed network camera shall support dynamic noise reduction to reduce bandwidth and storage requirements by optimizing the detail-to-bandwidth ratio via temporal and spatial noise filtering.

      - The fixed network camera shall be capable of capturing and storing images using H.265 and H.264 compression.

      - The fixed network camera shall deliver 6MP video, at rates up to 30 images per second.

    - System Features

      - The fixed network camera shall be compatible with the Bosch Video Client, Video Security client and the Bosch Video Management System.

      - The fixed network camera shall provide switchable audio input. This includes one (1) line-in level (10 kOhm typical, max. 1 Vrms) or one (1) integrated microphone array.

      - The fixed network camera shall provide one (1) audio line out (1.5 kOhm typical, 1 Vrms).

      - The fixed network camera shall provide one (1) alarm input and one (1) alarm output.

      - The fixed network cameras shall offer a connection speed option of 10/100/1000 MBps.

    - Pre-programmed Modes

      - The fixed network camera shall offer multiple pre-programmed configurable user modes for optimized settings for key applications.

      - The fixed network camera shall allow users to customize these modes for the specific requirements of the camera site.

    - Recording and Storage Management

      - The fixed network camera shall have a micro SD card slot that uses standard, off-the-shelf micro SD (Secure Digital), micro SDHC (Standard Digital High Capacity) or a micro SDXC (Secure Digital eXtended Capacity) card for local storage (up to 2 TB).

      - The fixed network camera shall support industrial SD cards with health monitoring.

      - The local storage feature shall be capable of storage for Automatic Network Replenishment (ANR).

      - The fixed network camera shall support iSCSI devices to allow video stream to be recorded directly to an iSCSI RAID array.

      - The fixed network camera shall support iSCSI storage targets.

      - The fixed network camera shall be compatible with the Bosch Video Recording Manager (VRM) to control and manage video recording.

    - Intelligent Video Analytics Pro (IVA Pro)

      - The fixed network camera shall offer embedded deep learning-based detection, and video analytics that eliminate dedicated PCs and associated software maintenance.

      - The camera shall use machine learning to allow users to define and classify objects of interest, both moving and non-moving, and then generate detectors for the objects. The machine learning program shall allow distribution of detectors to other cameras.

      - The fixed network camera shall offer advanced Intelligent Video Analytics Pro (IVA Pro) with automated calibration by use of an integrated gyro-sensor so only camera height needs to be set for a full calibration.

      - The fixed network camera shall offer IVA Pro Buildings Pack and IVA Pro Perimeter Pack with the following functionalities:

        - Triggers: any object, object in field, line crossing, enter field, leave field, loitering, follow route, idle object, removed object, counting, occupancy, crowd density, condition change, similarity search, flow, counter flow, conditional change

        - Filters: duration, size, aspect ratio, speed, direction, color, object classes

        - Object classes: Persons, Vehicles

        - Tracking modes: Perimeter tracking (2D), ship tracking, museum mode, Base tracking (2D)

      - The fixed network camera shall offer tamper detection: global change, reference check.

      - The fixed network camera shall support people counting.

      - The fixed network camera shall allow users to set up to two (2) separate profiles and switch profiles based on a day/night or holiday schedules.

      - The fixed network camera shall incorporate an IVA alarm rule engine that supports programming of alarm tasks which are individually reported to the video management software.

    - Access Security

      - The fixed network camera shall offer three levels of password protection.

      - The fixed network camera shall support 802.1x authentication using a RADIUS (Remote Authentication Dial In User Service) server.

      - The fixed network camera shall store a SSL certificate for use with HTTPS.

      - The fixed network camera shall support RSA encryption key lengths of up to 4096 bits.

      - The camera shall have advanced certificate handling with unique certificates and encrypted private keys.

      - The camera shall support secured connections.

      - The camera shall require a service-level password at initial setup to access camera functions.

      - The camera shall disable all unsecure ports.

      - The camera shall disable all unsecure remote communication.

      - The camera shall prevent uploading of untrusted, unauthenticated third-party software.

      - The camera shall support only authenticated firmware.

      - The camera shall incorporate an embedded login firewall.

      - The camera shall have software protection that seals the camera configuration (“software sealing”) and detects changes to camera configuration.

    - Installation Requirements

      - The fixed network camera shall support the following dual, redundant power options:

        - Input options:

          - PoE (IEEE 802.3af / 802.3at Type 1, Class 3)

          - 24 VAC

          - 12 VDC

      - The fixed network camera shall be able to be direct corner mounted to the ceiling of any room. The fixed network camera shall be capable of operating in an outdoor environment. The fixed network camera shall:

        - Offer IP66 environmental protection.

        - Offer IK10+ 50J impact resistance.

        - Conform to the UL50E type 4X standard.

      - The fixed network camera shall provide a multi-language on-screen display.

  - ACCESSORIES

    - Mounts & accessories

NDA-7080-CBB 	Conduit back box, corner

NDA-7080-FP 	Replacement face plate, corner

NDA-7080-IR940 	Replacement cover IR 940nm

NDA-7080-TC	Top cover, FLEXIDOME corner 7100i IR

NPD-3001-WAP 	Portable installation tool

NPD-5001-POE 	Midspan PoE Injector

NPD-5004-POE 	Midspan PoE Injector

MSD-064G 	IP SECURITY MICROSD CARD 64GB

MSD-128G		IP SECURITY MICROSD CARD 128GB

MSD-256G		IP SECURITY MICROSD CARD 256GB

    - Software options

      - MVC-IAA-GUN 	Gunshot Detector License, perpetual

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMONSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

202508081333