Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 CSI@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Intelligent Video Analytics (IVA) Pro Privacy

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

Intelligent Video Analytics (IVA) Pro Privacy

- GENERAL

  - SUMMARY

    - Section includes Intelligent Video Analytics (IVA) Pro Privacy, a video analytics software based on deep neural network technology for ensuring privacy while using another IVA Pro software that detects and tracks objects.

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 585 223 9180

Email: security.sales@us.bosch.com

www.boschsecurity.com

Bosch Security Systems, B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: +31 40 2577 284

Fax: +31 40 2577 330

Email: emea.securitysystems@bosch.com

www.boschsecurity.com

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

Email: apr.securitysystems@bosch.com

www.boschsecurity.com

    - Product Model:	MVC-IVA-PRI

  - GENERAL DESCRIPTION

    - The video analytics software shall ensure privacy while using another IVA Pro software that detects and tracks objects.

    - The video analytics software shall use a semi-transparent privacy mask, ensuring anonymization while still getting insights about moving objects in the area.

    - The video analytics software shall offer the option to mask the camera image fully or partially, while still allowing other IVA Pro software to run in these areas, obtaining anonymized metadata, counting statistics, and alarm events while protecting the privacy of the persons involved.

    - The video analytics software shall be able to dynamically mask all of the objects detected by other IVA Pro software, ensuring their privacy while giving the operator an unhindered view of the scene.

    - The video analytics software shall synchronize video streams and IVA Pro objects for the best anonymization performance.

    - The video analytics software shall allow users to configure IVA Pro Privacy settings per encoder stream, enabling the anonymization of persons in the live view while preserving evidence in a recorded stream. These privacy options shall be able to be combined in each stream to best suit the targeted application and required privacy levels.

    - The video analytics software shall allow users to select which IVA Pro objects they want to mask: all Pro objects, only faces, only persons, or only vehicles. The video analytics software shall for example allow users to decide that in their application it is enough to mask the person’s heads to protect their privacy while showing the rest of their bodies for production monitoring applications.

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

END OF SECTION

202411140904