IQSIGHT, formerly Bosch Video Systems, is a leading global supplier of video security so3lutions, backed by Bosch Engineering, that outsmart every potential threat to your safety and security.

For additional information, contact:

| Americas IQSIGHT LLC 130 Perinton Parkway Fairport, New York, 14450 USA Phone: +1-800-289-0096 Fax: +1-800-315-0470 video.systems@iqsight.com https://www.iqsight.com | Europe, Middle East, Africa IQSIGHT B.V. Achtseweg Zuid 173 5651 GW Eindhoven The Netherlands Phone: + 31 40 2577 202 Fax: +31 40 2577 330 video.systems@iqsight.com https://www.keenfinity-group.com | Asia-Pacific KEENFINITY PTE. LTD. 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 video.systems@iqsight.com apr.sales@keenfinity-group.com https://www.keenfinity-group.com |
| --- | --- | --- |

4MP OUTDOOR PTZ DOME CAMERA

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

4MP OUTDOOR PTZ DOME CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 4MP IP66-rated outdoor PTZ dome camera with H.264 and H.265 encoding, dynamic range of 133 dB, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

## Related Requirements

      - 27 15 00 Communications Horizontal Cabling

      - 27 20 00 Data Communications

      - 28 05 00 Common Work Results for Electronic Safety and Security

      - 28 06 20 Schedules for Video Surveillance

      - 28 16 15 Access Control Interfaces to Video Surveillance

      - 28 23 00 Video Management System

      - 28 31 31.15 Intrusion Detection Interfaces to Video Surveillance

      - 28 51 15.13 Information Interfaces to Video Surveillance Systems

  - REFERENCES

    - Abbreviations

      - AI – Artificial Intelligence

      - ARP – Address Resolution Protocol

      - DHCP - Dynamic Host Configuration Protocol

      - DNS - Domain Name Server

      - DDNS – Dynamic DNS

      - DORI - Detect, Observe, Recognize, Identify

      - fps - frames per second

      - HEVC – High Efficiency Video Coding

      - HTTPS – Secure Hypertext Transfer Protocol

      - ICMP - Internet Control Message Protocol

      - IP - Internet Protocol

      - IR – InfraRed

      - JPEG - Joint Photographic Experts Group

      - mDNS – Multicast DNS

      - MP - Megapixel

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - ONVIF - Open Network Video Interface Forum

      - PKI - Public Key Infrastructure

      - PoE - Power over Ethernet

      - RTCP – Real-Time Control Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - TCP - Transmission Control Protocol

      - TPM - Trusted Platform Module

      - TLS – Transport Layer Security

      - UDP - User Datagram Protocol

    - Reference Standards

      - Network

        - IEEE 802.3 Ethernet Standards

      - Video

        - ITU H.264

        - H.265/HEVC

        - MJPEG

      - ONVIF

        - EN 50132-5-2

      - Emissions/Immunity

        - FCC CFR 47 part 15 (Class A)

        - EN 55032 - Electromagnetic compatibility of multimedia equipment. Emission Requirements – Class A

        - ICES-003, Class A

        - EN 50130-4

        - EN 50121-4 (Railway applications)

        - CISPR 32 (Class A)

      - Safety

        - EN 62368-1 Ed.2 & Ed.3

        - IEC 62368-1 Ed.2 & Ed.3

        - UL 62368-1 Ed.3

        - IEC 60950-22 (needed for Ed.3 of IEC / EN 62368-1 Ed.2)

        - CAN/CSA-C22.2 No. 62368-1-19, Ed.3

      - Environmental

        - Water/dust protection / Impact protection

          - EN 60529 – Degrees of protection provided by enclosures – IP66

          - UL Type 4X

          - IEC 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts – IK10

      - Cybersecurity

        - IEC 62443-1-1

        - IEC 62443-2

        - FIPS 140-2 Level 3

        - FIPS 140-3 Level 3

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

      - Shop Drawings

      - Cyber hardening guidance

    - Closeout Submittals

      - Final listing of devices and settings

      - System test results

      - Statement of compliance with Manufacturer Cyber Hardening Guidelines

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The AUTODOME family of products is covered by a limited hardware warranty period of 3 years for moving parts and 5 years for non-moving parts, from the date of shipment, against any proved defect in materials or workmanship.

This warranty will be limited to a period of one year from the date of original purchase for moving parts such as, but not limited to:

      - Pan/Tilt drive and belts

      - Electrical slip-ring contacts

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	IQSIGHT LLC

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800-289-0096

Fax: +1 800-315-0470

Email: video.systems@iqsight.com

https://www.iqsight.com

IQSIGHT B.V.
Achtseweg Zuid 173
5651 GW Eindhoven
The Netherlands

Phone: + 31 40 2577 202

Fax: +31 40 2577 330

video.systems@iqsight.com

https://www.keenfinity-group.com

KEENFINITY PTE. LTD.

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

video.systems@iqsight.com  
apr.sales@keenfinity-group.com

https://www.keenfinity-group.com

    - Product Model:	NDP-7803S-Z30

  - GENERAL DESCRIPTION

    - The PTZ dome camera (“dome camera)” shall be a 4MP IP66-rated outdoor dome camera with H.264 and H.265 encoding, dynamic range of 133 dB, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

    - The dome camera shall possess the following primary characteristics:

      - H.264, H.265, and MJPEG compression

      - Resolution up to 2560 x 1440 pixels

      - Frame rate up to 60 fps

      - Suite of video analytics based on Artificial Intelligence and deep learning

      - Uni-directional audio

      - DORI technology to facilitate distinguishing of persons or objects

      - Multiple data security elements including 4096-bit RSA encryption, 512-bit ECC elliptic curves; AES/CBC 256 bit, and update capability for future software security functions.

      - Available posting to cloud-based services

      - Pre-configured optimized scene modes

      - Remote camera management

      - Mobile device camera setup and configuration

      - IP66, UL Type 4X, and IK10 environmental ratings

      - Power:	24 VAC, 36 VDC, or PoE IEEE 802.3bt, Type 3, Class 6

  - ADDITIONAL VIDEO/CAMERA SPECIFICATIONS

    - Imaging Device:

      - Size:		1/1.8 inch

      - Resolution:	4MP

    - Lens:

      - Focal Length: 	6.6 mm – 198 mm

      - Aperture:	F/1.5 – F/4.8

      - Horizonal Field of view:		58.7° – 2.2°

      - IR corrected

      - Day/Night:	Switchable IR-cut filter

    - Video Streaming:

      - Compression type:	H.264, MJPEG, H.265/HEVC

      - Number of streams: 	4 streams

      - Resolutions:

        - 2560 x 1440

        - 1920 x 1080

        - 1280 x 720

        - 960 x 544

        - 640 x 360

        - 520 x 184

        - 480 x 272

      - Frame rate:		1 - 60 fps

      - Dynamic range:		133 dB

    - Additional Video Functions:

      - Day/night modes: Auto (adjustable switch points); Color; Monochrome

      - Backlight compensation

      - Display stamping

      - Electronic image stabilization

      - White balance:

        - 2000 K – 10,000 K

        - Modes

          - Basic auto

          - Standard auto

          - Manual RGB

          - Sodium lamp auto

      - Number of privacy masks:  32

      - Privacy mask patterns:

        - Black

        - White

        - Gray

        - Auto

        - Custom Color (16M colors)

      - Minimum illumination (color):  		0.0168 lx

      - Minimum illumination (monochrome):  	0.0017

    - Video Content Analysis:

      - Embedded deep learning-based detection and video analytics (IVA Pro) eliminate the need for dedicated PCs and associated software maintenance.

      - Embedded IVA Pro Buildings, IVA Pro Perimeter, and IVA Pro Privacy provide reliable deep learning-based detection.

      - Other license options include:

        - IVA Pro Traffic

        - IVA Pro Intelligent Tracking

        - IVA Pro Visual Gun Detection

        - IVA Pro Appearance

        - IVA Pro Protective Equipment (IVA Pro PPE)

      - GenAI+:	Yes (available Q4 2026)

      - Comprehensive video content analysis capabilities include detection and classification of Object types:  persons and vehicles.

      - A wide range of video analytics rule configuration (alarm triggers) include:

        - Any object

        - Object in field

        - Line crossing

        - Enter / leave field

        - Loitering

        - Follow route

        - Idle / removed object

        - Counting

        - Occupancy

        - Condition change

        - Similarity search

        - Tampering

        - Rule based alarms and tracking

        - Audio detection (May also be used to generate an alarm.)

      - Camera Trainer allows users to train the system for custom object detection and classification.

    - Local SD storage

      - Internal storage: MicroSDHC; MicroSDXC; MicroSDSC; 5-s-pre-alarm-recording

      - Memory card capacity: Up to 2TB

    - Audio

      - Line in

      - Line out

      - Compression:

        - G.711 8 kHz

        - L16 16 kHz

        - AAC

        - AAC-LC 48kbps 16 kHz

        - AAC-LC 80kbps 16 kHz

      - Streaming:  Uni-directional

    - ONVIF Conformity:

      - Profile S

      - Profile G

      - Profile T

      - Profile M

  - NETWORK

    - Connectivity:  100BASE-TX/1000BASE-T Ethernet

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Internet Control Message Protocol (ICMP) and ICMP v6, Hypertext Transfer Protocol (HTTP), Secure Hypertext Transfer Protocol (HTTPS)

      - Network services: Address Resolution Protocol (ARP), Domain Name System (DNS, DNSv6), Dynamic DNS (DDNS), Network Time Protocol (NTP), File Transfer Protocol (FTP), System Network Management Protocol (SNMP) v3 and MIBII

      - Media: Real-Time Control Protocol (RTCP), Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP); Internet Group Messaging Protocol (IGMP)

      - Security: IEEE 802.1x Extensible Authentication Protocol (EAP)/Transport Layer Security (TLS)

    - Data Security

      - Crypto coprocessor: 	RSA 3072 bit for TLS; RSA 4096 bit for device identity; AES/CBC 256 bit

      - Advanced handling certificates

          - Manufacturer origin device certificate pre-installed

          - Self-signed unique certificates automatically generated when required

      - End-to-end encryption: 	Full end-to-end with supported VMS

      - Encryption 		TLS 1.2; TLS 1.3; AES 256; AES 128

      - Video Authentication:	MD5; SHA-1; SHA-256; Watermark

      - Local storage encryption: XTS-AES

      - Firmware protection:	Signed firmware, secure boot; software sealing

  - ELECTRICAL

    - Power Input:

      - 24 VAC (IEC 62368-1 Limited Power Source)

      - 36 VDC (IEC 62368-1 Limited Power Source)

      - PoE:

        - Full-function mode:  IEEE 802.3bt Type 3, Class 6, 60W

        - Low-power mode:  IEEE 802.3bt Type 2, Class 4, 30W

    - Power Consumption (typical – maximum)

      - 24 VAC:	32 VA – 72 VA at 24VAC, 50/60Hz

      - 36 VDC:	17 W – 46.8 W

      - PoE:		21 W – 48.6 W at PoE 54VDC (IEEE 802.3bt Type 3, Class 6)

    - Build-in Surge Suppression

      - Built-in surge protection for power, data, and network interfaces

      - Compliant with EN 55024 levels. Note: The camera must be installed per the installation manual to achieve the standard levels.

  - MECHANICAL AND ENVIRONMENTAL

    - Construction Material: 	Aluminum housing; polycarbonate bubble

    - Finish:			White

    - Impact Resistance:		IK10

    - Ingress Protection:		IP66

    - Dimensions (D x H):  	210.65 mm x 324 mm (8.3 in. x 12.8 in.)

    - Temperature:

      - Operating: 	 	-40 °C to 60 °C (-40 °F to 140 °F)

      - Storage:		-40 °C to 70 ºC (-40 °F to 158 ºF)

    - Relative Humidity (non-condensing):

      - Operating		0 to 95%

      - Storage			0 to 95%

    - Mounting:

      - Pendant arm mount

      - Pipe mount

    - Wind load:		Camera will hold its position in gusts up to 274 kph (170 mph).

    - Mean time between failures (MTBF):		~276,000 hours

  - ACCESSORIES

    - The Manufacturer shall offer accessories to enable the mounting of the dome camera in the following circumstances: Pendant – wall or pipe

    - Bubbles

      - NDA-7100-CBL		Clear replacement bubble

      - NDA-7100-TBL		Tinted replacement bubble

      - NDA-7100-HCBL	Clear, Hydrophilic replacement bubble

    - Mid-span injectors

      - NPD-6001C		60W indoor

      - NPD-6001C-E		60W IP67

      - NPD-6001-I		60W indoor, industrial

      - NPD-6001C-BT		60W indoor TAA

      - NPD-6001C-EBT	60W IP67 TAA

      - NPD-6001-IBT		60W indoor, industrial TAA

      - NDA-6090-PMA		pole mount for midspan, outdoor

    - Mounting accessories and power supplies

      - NDA-7100-PA0H	Wall arm & PSU, 24VAC

      - NDA-7100-PA0HF	Wall arm & PSU with fiber connection, 24VAC

      - NDA-7100-PA1H	Wall arm & PSU, 120VAC

      - NDA-7100-PA1HF	Wall arm & PSU with fiber connection, 120VAC

      - NDA-7100-PA2H	Wall arm & PSU, 230VAC

      - NDA-7100-PA2HF	Wall arm & PSU with fiber connection, 230VAC

      - NDA-U-PSU0H		PSU, IR, 24VAC

      - NDA-U-PSU1H		PSU, IR, 120VAC

      - NDA-U-PSU2H		PSU, IR, 230VAC

      - NDA-7100-PEN		Pendant arm

      - NDA-7100-PENF	Pendant arm with fiber connection

      - NDA-7100-PIPE	Pipe mount

      - NDA-7100-PIPEF	Pipe mount with fiber connection

      - NDA-PEND-WPLATE	Mounting plate for NDA-7100-PEN/PENF

      - NDA-U-WMP		Wall mount plate

      - NDA-U-CMT		Corner mount adapter

      - NDA-U-PMT		Pendant pipe mount, 12" (31cm)

      - NDA-U-PMAL		Pole mount adapter large

      - NDA-U-RMT		Pendant parapet mount

      - LTC 9230/01		Parapet roof mount adapter

      - NDA-7100-WP		Weather protector

    - Industrial microSD cards

      - MSD-064G		microSD card 64GB

      - MSD-128G		microSD card 128GB

      - MSD-256G		microSD card 256GB

    - Software Options

      - MVC-IVA-TRA		IVA Pro Traffic

      - MVC-IVA-IT		IVA Pro Intelligent Tracking

      - MVC-IVA-APC		IVA Pro Appearance

      - MVC-IVA-PPE		IVA Pro PPE

      - MVC-IVA-VGD		IVA Pro Visual Gun Detection

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

  - STORAGE

    - Products shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

202605261100