Bosch Security Systems is a leading global supplier of security, safety, and communications products and systems. Protecting lives, buildings, and assets is the major aim. The product portfolio includes video security, intrusion detection, and fire detection as well as access control and management systems.

For additional information, contact:

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450 USA Phone: +1-800-289-0096 Fax: +1-800-315-0470 presales.support@keenfinity-group.com https://www.keenfinity-group.com/ | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 security.sales@keenfinity-group.com https://www.keenfinity-group.com/ | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.sales@keenfinity-group.com https://www.keenfinity-group.com/ |
| --- | --- | --- |

8MP OUTDOOR PTZ DOME CAMERA with IR

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold bracketed text>.

2.  Explanatory notes and comments are presented in italicized colored text.

8MP OUTDOOR PTZ DOME CAMERA with IR

- GENERAL

  - SUMMARY

    - Section includes an 8MP IP66-rated outdoor PTZ dome camera with integrated, long-life Illuminator, H.264 and H.265 encoding, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

## Related Requirements

      - 27 15 00 Communications Horizontal Cabling

      - 27 20 00 Data Communications

      - 28 05 00 Common Work Results for Electronic Safety and Security

      - 28 06 20 Schedules for Video Surveillance

      - 28 16 15 Access Control Interfaces to Video Surveillance

      - 28 23 00 Video Management System

      - 28 31 31.15 Intrusion Detection Interfaces to Video Surveillance

      - 28 51 15.13 Information Interfaces to Video Surveillance Systems

  - REFERENCES

    - Abbreviations

      - AI – Artificial Intelligence

      - ARP – Address Resolution Protocol

      - DHCP - Dynamic Host Configuration Protocol

      - DNS - Domain Name Server

      - DDNS – Dynamic DNS

      - DORI - Detect, Observe, Recognize, Identify

      - fps - frames per second

      - HEVC – High Efficiency Video Coding

      - HTTPS – Secure Hypertext Transfer Protocol

      - ICMP - Internet Control Message Protocol

      - IP - Internet Protocol

      - IR – InfraRed

      - JPEG - Joint Photographic Experts Group

      - mDNS – Multicast DNS

      - MP - Megapixel

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - ONVIF - Open Network Video Interface Forum

      - PKI - Public Key Infrastructure

      - PoE - Power over Ethernet

      - RTCP – Real-Time Control Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - TCP - Transmission Control Protocol

      - TPM - Trusted Platform Module

      - TLS – Transport Layer Security

      - UDP - User Datagram Protocol

    - Reference Standards

      - Network

        - IEEE 802.3 Ethernet Standards

      - Video

        - ITU H.264

        - H.265/HEVC

        - MJPEG

      - ONVIF

        - EN 50132-5-2

      - Emissions

        - FCC CFR 47 part 15 (Class A)

        - EN 55032 - Electromagnetic compatibility of multimedia equipment. Emission Requirements – Class A

        - ICES-003, Class A

        - EN 50130-4

        - EN 50121-4 (Railway applications)

        - CISPR 32 (Class A)

      - Safety

        - EN 62368-1 Ed.2 & Ed.3

        - IEC 62368-1 Ed.2 & Ed.3

        - UL 62368-1 Ed.3

        - IEC 60950-22 (needed for Ed.3 of IEC / EN 62368-1 Ed.2)

        - CAN/CSA-C22.2 No.  62368-1-19

        - IEC 62471

      - Environmental

        - IEC

          - EN 60529 – Degrees of protection provided by enclosures – IP66

          - UL Type 4X

          - IEC 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts – IK10

      - Cybersecurity

        - FIPS 140-3

  - SUBMITTALS

    - Informational Submittals

      - Product Data

        - Manufacturer’s printed or electronic data sheets

        - Manufacturer’s installation and operation manuals

        - Warranty documentation

      - Shop Drawings

      - Cyber hardening guidance

    - Closeout Submittals

      - Final listing of devices and settings

      - System test results

      - Statement of compliance with Manufacturer Cyber Hardening Guidelines

  - QUALIFICATIONS

    - Installers shall be certified, trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The AUTODOME family of products is covered by a limited hardware warranty period of 3 years for moving parts and 5 years for non-moving parts, from the date of shipment, against any proved defect in materials or workmanship.

This warranty will be limited to a period of one year from the date of original purchase for moving parts such as, but not limited to:

      - Pan/Tilt drive and belts

      - Electrical slip-ring contacts

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450

USA

Phone: +1 800 289 0096

Fax: +1 800-315-0470

Email: presales.support@keenfinity-group.com

    - Product Model:	NDP-7804-Z12L

  - GENERAL DESCRIPTION

    - The PTZ dome camera (“dome camera)” shall be an 8MP IP66-rated outdoor dome camera with integrated, long-life illuminator, H.264 and H.265 encoding, and built-in artificial intelligence providing full situational awareness and triggers for relevant alerts.

    - The dome camera shall possess the following primary characteristics:

      - H.264, H.265, and MJPEG compression

      - Resolution up to 3840 x 2160 pixels

      - Frame rate up to 30 fps

      - Suite of video analytics based on Artificial Intelligence and deep learning

      - Uni-directional audio

      - DORI technology to facilitate distinguishing of persons or objects

      - Multiple data security elements including Trusted Platform Module (TPM), PKI, 4096-bit RSA encryption, and update capability for future software security functions.

      - Available posting to cloud-based services

      - Pre-configured optimized scene modes

      - Remote camera management

      - Mobile device camera setup and configuration

      - IP66, UL Type 4X, and IK10 environmental ratings

      - Power:	24 VAC, 36 VDC, or PoE+ IEEE 802.3bt, Type 4, Class 8

  - ADDITIONAL VIDEO/CAMERA SPECIFICATIONS

    - Imaging Device:

      - Size:		1 inch

      - Resolution:	8MP

    - Lens:

      - Focal Length: 	9.3 – 111.6 mm

      - Aperture:	F/2.8 – F/4.5

      - Horizontal Field of view:	6.1° x 64.6°

      - Zoom/focus:	Automatic (with manual override)

      - Iris: 		Automatic (with manual override)

      - IR corrected

      - Day/Night:	Switchable IR-cut filter

    - Video Streaming:

      - Compression type:	H.264, MJPEG, H.265 HEVC

      - Number of streams: 	4 streams

      - Resolutions:

        - 3840 x 2160

        - 1920 x 1080

        - 1536 x 864

        - 1280 x 720

        - 720 x 576

        - 704 x 576

        - 720 x 480

        - 704 x 480

        - 768 x 432

        - 512 x 288

        - 640 x 480

      - Frame rate:		1 - 30 fps

    - Night vision (Illumination):

      - LED wavelengths:	850 nm

      - Detection distance:	200 m (656 ft)

      - IR intensity:		Manually adjustable; Automatic

      - IR horizontal beam angle (°FWHM)

        - Narrow	9.4 deg FWHM

        - Medium	25.7 deg FWHM

        - Wide	66.9 deg FWHM

    - Additional Video Functions:

      - Day/night modes: Auto (adjustable switch points); Color; Monochrome

      - Backlight compensation

      - Video watermarking

      - Display stamping

      - Location

      - Optical image stabilization

      - White balance:

        - 2000 K – 10,000 K

        - Modes

          - ATW

          - Extended ATW

          - AWB hold

          - Indoor

          - Outdoor

          - Manual

      - Number of privacy masks:  32

      - Privacy mask patterns:

        - Black

        - White

        - Gray

        - Auto color

        - 16M colors

        - Mosaic

        - 10 blurred levels

      - Minimum illumination (monochrome): 0.1430 lx

      - Minimum illumination (color): 0.2998 lx

      - Video Content Analysis:

        - Object types: person or vehicle

        - Alarm Triggers

          - Any object

          - Object in field

          - Line crossing

          - Enter / leave field

          - Loitering

          - Follow route

          - Idle /Removed object

          - Counting

          - Occupancy

          - Condition change

          - Similarity search

          - Tampering

          - Rule based alarms and tracking

          - Audio detection (May also be used to generate an alarm.)

    - Local SD storage

      - SDHC; SDXC; SDSC

      - Memory card capacity up to 2TB

      - Recording modes:  Manual; Scheduled; Pre-alarm; Alarm

    - Audio

      - Line in

      - Line out

      - Compression:

        - G.711 8 kHz

        - L16 16 kHz

        - AAC-LC

        - AAC-LC 48kbps 16 kHz

        - AAC-LC 80kbps 16 kHz

      - Streaming:  Uni-directional

    - ONVIF Conformity:

      - Profile S

      - Profile G

      - Profile T

      - Profile M

  - NETWORK

    - Connectivity: 100BASE-TX/1000BASE-T Ethernet

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Internet Control Message Protocol (ICMP) and ICMP v6, Hypertext Transfer Protocol (HTTP), Secure Hypertext Transfer Protocol (HTTPS)

      - Network services: Address Resolution Protocol (ARP), Domain Name System (DNS, DNSv6), Dynamic DNS (DDNS), Network Time Protocol (NTP), File Transfer Protocol (FTP), System Network Management Protocol (SNMP) v3 and MIBII

      - Media: Real-Time Control Protocol (RTCP), Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP); Internet Group Messaging Protocol (IGMP), DiffServ (QoS)

      - Security: IEEE 802.1x Extensible Authentication Protocol (EAP)/Transport Layer Security (TLS)

    - Data Security

      - Crypto coprocessor: 	RSA 3072 bit for TLS; RSA 4096 bit for device identity; AES/CBC 256 bit

      - Advanced handling certificates

          - Manufacturer origin device certificate pre-installed

          - Self-signed unique certificates automatically generated when required

      - End-to-end encryption: 	Full end-to-end with supported VMS

      - Encryption 		TLS 1.2; TLS 1.3; AES 256; AES 128

      - Video Authentication:	MD5; SHA-1; SHA-256; Watermark

      - Local storage encryption: XTS-AES

      - Firmware protection:	Signed firmware, secure boot; software sealing

  - ELECTRICAL

    - Power Input:

      - 24 VAC

      - 36 VDC

      - PoE:	IEEE 802.3bt Type 4, Class 8 (4PPOE) 90W at PSE (POE midspan or POE Ethernet switch) side, (71.3W at PTZ camera AKA PD side) using a single-signature PD (using the 90 W midspan)

    - Power Consumption (maximum)

      - 24 VAC:	53W

      - 36 VDC:	66.6W

      - PoE:		64.8W at PoE 54VDC (IEEE 802.3bt Type 4, Class 8)

    - Current Consumption VAC (A) (typical – maximum)

      - 24 VAC:	3.5A

      - 36 VDC:	1.2A

      - PoE:		1.85A

    - Build-in Surge Suppression

      - Compliant with EN 55024 levels. Note: The camera must be installed per the installation manual to achieve the standard levels.

  - MECHANICAL AND ENVIRONMENTAL

    - Construction Material: 	Aluminum housing; polycarbonate bubble

    - Finish options:		White

    - Impact Resistance:		IK10

    - Dimensions (D x H):  	210.65 mm (8.3 in.) x 324 mm (4.8 in.)

    - Temperature:

      - Operating (IR on): 	-40 °C to 50 °C (-40 °F to 122 °F)

      - Operating (IR off): 	-40 °C to 60 °C (-40 °F to 140 °F)

      - Storage:		-40°C to 70ºC (-40°F to 158ºF)

    - Relative Humidity (non-condensing):

      - Operating:		0 to 95%

      - Storage	:		0 to 95%

    - Ingress Protection:		IP66

    - Mounting:

      - Pendant arm mount

      - Pipe mount

    - Wind load:		Camera will hold its position in gusts up to 274 kph (170 mph).

    - Mean time between failures (MTBF):		131,500 hours

  - ACCESSORIES

    - The Manufacturer shall offer accessories to enable the mounting of the dome camera in the following circumstances:

      - Pendant – wall or pipe

    - Other Accessories

      - Mid-span injectors

      - Spare bubble

      - Industrial SD cards – 64 GB, 128 GB, 256 GB

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

    - Before permanent installation of the system, the Contractor shall test the system in conditions simulating the final installed environment.

  - INSTALLATION

    - The contractor shall follow all Manufacturer-published guidance on proper installation and configuration of the installed cameras.

    - The Contractor shall test the system in conditions simulating the final installed environment

    - Reports:

      - System test results

      - System network addressing and device settings

  - STORAGE

    - Products shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION

202508221215